# VirtualBox Network Troubleshooting Lab

A small virtual networking lab built using Oracle VirtualBox, Windows virtual machines, and Ubuntu Linux.

This lab was created as part of my studies in Certificate IV in Cyber Security and my ongoing CCNA studies. The goal was to practise basic network configuration, connectivity testing, and troubleshooting in an isolated virtual environment.

## Lab Overview

In this lab, I created a small virtual network containing:

- Computer01 — Windows client

- Computer02 — Windows client

- Ubuntu — Ubuntu Linux VM acting as the default gateway

- CiscoLab — VirtualBox Internal Network connecting the three VMs

The lab uses static IPv4 addressing and tests connectivity between the virtual machines using ping.

The final troubleshooting scenario involves identifying a Windows Defender Firewall rule that was preventing ICMP Echo Requests from receiving a response.

## Network Topology

Virtual Network

CiscoLab

│

┌─────────────┼─────────────┐

│             │             │

Computer01     Computer02     Ubuntu

Windows         Windows        Linux

Client          Client         Gateway

│             │             │

└─────────────┴─────────────┘

192.168.10.10   192.168.10.20   192.168.10.1

All three virtual machines are connected to the same 192.168.10.0/24 network.

## IP Addressing

| Device | Operating System | Role | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- | --- |
| Computer01 | Windows | Client | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| Computer02 | Windows | Client | 192.168.10.20 | 255.255.255.0 | 192.168.10.1 |
| Ubuntu | Ubuntu Linux | Gateway | 192.168.10.1 | 255.255.255.0 | — |

The lab configuration assigns Ubuntu 192.168.10.1, Computer01 192.168.10.10, and Computer02 192.168.10.20, all using a /24 subnet.

## VirtualBox Configuration

Each virtual machine was configured using:

- Network Adapter: Adapter 1

- Attached To: Internal Network

- Internal Network Name: CiscoLab

- Promiscuous Mode: Deny

- Virtual Cable: Connected

Using an Internal Network creates an isolated virtual network allowing the VMs to communicate with each other without requiring an external physical network connection.

## Configuration

### Ubuntu

Ubuntu was configured with:

IP Address: 192.168.10.1

Subnet Mask: 255.255.255.0

Gateway: Not configured

The configuration was verified using:

ip addr

### Computer01

Computer01 was configured with:

IP Address: 192.168.10.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.10.1

The configuration was verified using:

ipconfig

### Computer02

Computer02 was configured with:

IP Address: 192.168.10.20

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.10.1

The configuration was verified using:

ipconfig

# Connectivity Testing

After configuring the machines, I used ping to test connectivity.

### Computer01 → Ubuntu

ping 192.168.10.1

Result: PASS

### Computer02 → Ubuntu

ping 192.168.10.1

Result: PASS

### Computer01 → Computer02

ping 192.168.10.20

Result: Initial failure

### Computer02 → Computer01

ping 192.168.10.10

Result: Initial failure

The lab specifically uses these connectivity tests to verify communication between the clients and the Ubuntu gateway, as well as communication between the two Windows clients.

# Troubleshooting

## Problem

Although the IP addresses, subnet masks, and default gateways were configured correctly, the Windows machines were initially unable to successfully ping each other.

The result included:

Request timed out.

Instead of immediately changing the network configuration, I worked through the problem systematically.

### Initial checks

| Check | Result |
| --- | --- |
| IP address configuration | ✅ Correct |
| Subnet mask | ✅ Correct |
| Default gateway | ✅ Correct |
| VirtualBox Internal Network | ✅ Correct |
| Computer01 → Ubuntu | ✅ Working |
| Computer02 → Ubuntu | ✅ Working |
| Computer01 → Computer02 | ❌ Failed |
| Computer02 → Computer01 | ❌ Failed |

Because the clients could communicate with the Ubuntu gateway, the basic virtual network configuration appeared to be functioning.

# Windows Defender Firewall

I investigated Windows Defender Firewall with Advanced Security.

Under:

Inbound Rules

I located:

File and Printer Sharing

(Echo Request - ICMPv4-In)

The rule was disabled.

The lab identifies Windows Firewall blocking incoming ICMP Echo Requests as the cause of the failed ping tests.

## Solution

Rather than completely disabling Windows Defender Firewall, I enabled the specific inbound rule:

File and Printer Sharing

(Echo Request - ICMPv4-In)

After enabling the rule, I repeated the ping test:

ping 192.168.10.20

The test then returned successful replies.

Reply from 192.168.10.20

The same process was used to verify connectivity in the opposite direction.

# Verification

After troubleshooting, the final connectivity tests were successful:

| Source | Destination | Result |
| --- | --- | --- |
| Computer01 | Ubuntu 192.168.10.1 | ✅ PASS |
| Computer02 | Ubuntu 192.168.10.1 | ✅ PASS |
| Computer01 | Computer02 192.168.10.20 | ✅ PASS |
| Computer02 | Computer01 192.168.10.10 | ✅ PASS |

# What I Learned

This lab helped me practise:

- Configuring VirtualBox Internal Networks

- Understanding basic virtual networking

- Configuring static IPv4 addresses

- Understanding /24 subnetting

- Configuring default gateways

- Using ipconfig to troubleshoot Windows networking

- Using ip addr to verify Linux network configuration

- Using ping to test network connectivity

- Understanding ICMP Echo Requests and Echo Replies

- Troubleshooting Windows Defender Firewall

- Understanding inbound firewall rules

- Applying the principle of least privilege

- Documenting a troubleshooting process

The lab's stated learning outcomes include configuring Internal Networks, static IPv4 addressing, using ipconfig and ping, identifying Windows Firewall issues, and documenting troubleshooting processes.

# Security Consideration

A key lesson from this lab was that the easiest solution isn't always the best solution.

It would have been possible to completely disable Windows Defender Firewall to allow the ping traffic.

Instead, I enabled only the specific ICMPv4 Echo Request rule required for the test.

This follows the principle of least privilege by allowing the required traffic without unnecessarily disabling the host firewall.

# Tools Used

- Oracle VirtualBox

- Windows

- Ubuntu Linux

- Windows Command Prompt

- Linux Terminal

- ipconfig

- ip addr

- ping

- Windows Defender Firewall with Advanced Security

# Learning Context

This project was completed as part of my ongoing development in:

Certificate IV in Cyber Security

and

CCNA studies

I'm using home labs like this to reinforce the networking concepts I'm learning and to gain practical experience with configuring and troubleshooting networks.

# Future Improvements

This lab is a basic starting point. Future labs will expand the environment and introduce more advanced networking concepts, including:

- Multiple subnets

- Routing between networks

- VLANs

- DHCP

- DNS

- Windows Server

- Linux networking

- Packet analysis with Wireshark

- Network security controls

- Firewall configuration

- Network troubleshooting scenarios

The goal is to progressively build more complex networking and cybersecurity environments rather than relying solely on theoretical study.

## Disclaimer

This is a personal educational lab created for learning and experimentation in an isolated virtual environment.

The configurations and techniques demonstrated here should be tested in controlled environments and should not be applied to production systems without understanding their security implications.

### Software & ISO Downloads

Oracle VirtualBox

https://www.virtualbox.org/wiki/Downloads

Ubuntu Desktop

https://ubuntu.com/download/desktop

Windows 11 ISO

https://www.microsoft.com/en-ca/software-download/windows11

Windows 10

https://www.microsoft.com/en-ca/software-download/windows10

Note: Windows 10 reached end of free support on October 14, 2025. For new labs, Windows 11 may be preferable where your hardware and VirtualBox setup support it.